SECURING QUANTUM CHANNEL

Research Analysis: BB84 Protocol, QBER & Quantum Temporal Authentication

Quantum Key Distribution • Man-in-the-Middle Detection • Nanosecond-Scale Authentication

# Executive Summary

This report examines the BB84 Quantum Key Distribution (QKD) protocol — the first and most widely deployed quantum cryptographic scheme — and two complementary mechanisms for detecting adversarial interference: the Quantum Bit Error Rate (QBER) and Quantum Temporal Authentication (QTA). Together, these form a multi-layered security architecture that can detect eavesdropping attempts with information-theoretic certainty, grounded in the laws of quantum mechanics rather than computational hardness assumptions.

Core Security Guarantee: Any eavesdropper (Eve) attempting to intercept qubits must measure them — but quantum measurement irreversibly disturbs the quantum state. This disturbance manifests as a measurable error rate (QBER) and an unavoidable propagation delay (detectable by QTA), making eavesdropping both statistically and physically detectable.

# Part 1: The BB84 Protocol

## 1.1  Historical Context & Overview

BB84 was proposed in 1984 by Charles Bennett and Gilles Brassard, making it the world’s first quantum cryptographic protocol. Unlike classical key exchange (e.g., Diffie-Hellman), BB84 derives its security not from mathematical complexity but from a fundamental principle of quantum physics: measuring an unknown quantum state inevitably disturbs it.

The protocol enables two parties, conventionally named Alice (sender) and Bob (receiver), to establish a shared secret key over an insecure quantum channel, with any eavesdropping attempt leaving detectable traces in the transmission statistics.

## 1.2  Quantum Encoding: Two Non-Orthogonal Bases

BB84’s security hinges on using two incompatible measurement bases. Because the bases are non-orthogonal (neither is a subset of the other), it is physically impossible for an observer to determine which basis was used for encoding without a 50% chance of disturbing the qubit’s state.

| Basis | Bit 0 Encoding | Bit 1 Encoding | Measurement Rule |
| --- | --- | --- | --- |
| Rectilinear (+) | ¦0⟩ (Horizontal, 0°) | ¦1⟩ (Vertical, 90°) | Correct basis → deterministic result |
| Diagonal (×) | ¦+⟩ (45° diagonal) | ¦−⟩ (135° diagonal) | Wrong basis → random collapse (50%) |

The critical property exploited by BB84 is the Heisenberg Uncertainty Principle: if Eve measures a qubit in the wrong basis, she collapses the superposition to a random eigenstate. When she then re-prepares and retransmits the qubit, it no longer faithfully represents Alice’s original bit in ~50% of those cases.

## 1.3  Protocol Phases Step-by-Step

| # | Phase | Description |
| --- | --- | --- |
| 1 | Qubit Preparation | Alice encodes each random bit using a randomly chosen basis (+ or ×) and transmits photons over the quantum channel. |
| 2 | Quantum Transmission | Photons travel over fiber or free-space. No classical information is exchanged at this stage. |
| 3 | Bob’s Measurement | Bob independently selects a random measurement basis for each incoming photon. |
| 4 | Basis Sifting | Over a public classical channel, Alice and Bob announce their basis choices (not the bits). They discard all positions where bases differed (~50%). |
| 5 | QBER Estimation | A random sample of the sifted key is compared publicly. Error rate = QBER. If QBER ≥ 11%, the session is aborted. |
| 6 | Privacy Amplification | If QBER is safe, the remaining sifted bits form the raw key, further condensed via hashing to remove Eve’s partial knowledge. |

Key Insight: After sifting, Alice and Bob hold identical bit strings — unless someone interfered. The QBER of a perfect, noiseless, eavesdrop-free channel is 0%. Any deviation above ~2–3% warrants investigation; above 11% mandates session abort.

## 1.4  The No-Cloning Theorem: Why Passive Eavesdropping Fails

A natural question is whether Eve could simply copy the qubit without measuring it, read the copy, and forward the original undisturbed. The No-Cloning Theorem (Wootters & Zurek, 1982) proves this is physically impossible. An arbitrary unknown quantum state cannot be duplicated. Eve must either measure-and-resend (introducing errors) or perform a more sophisticated entanglement-based attack — which still leaves detectable signatures in the QBER.

# Part 2: Quantum Bit Error Rate (QBER)

## 2.1  Definition & Measurement

The Quantum Bit Error Rate (QBER) is the fraction of sifted key bits that differ between Alice and Bob after basis reconciliation. It is measured by publicly sacrificing a random sample of the sifted key:

QBER  =  (Number of mismatched bits in sample)  ÷  (Total sample size)

This sample is consumed during error estimation and cannot be used as key material. A larger sample improves QBER accuracy but reduces the final key length — standard implementations use 25–50% of the sifted key for this step.

## 2.2  QBER Under Eve’s Intercept-Resend Attack

When Eve intercepts a fraction p of transmitted qubits and re-sends them after measurement:

- Basis Match (50% of interceptions): Eve guesses the correct basis. She reads the correct bit and re-sends an identical qubit. No error is introduced for these cases.

- Basis Mismatch (50% of interceptions): Eve guesses the wrong basis. The qubit collapses to a random state. When Bob measures it, he gets the wrong bit 50% of the time. Error probability for this subset: 50%.

Overall QBER contribution from Eve: p × 0.5 × 0.5 = p × 0.25

This gives the fundamental relationship that underpins BB84’s security:

QBER = 0.25 × (Eve’s interception rate)     ⟹     Abort threshold 11% ⇒ Eve intercepting ≥44%

## 2.3  QBER Threshold Decision Framework

| QBER Range | Interpretation | Action |
| --- | --- | --- |
| 0–2% | Negligible channel noise | Secure — proceed to privacy amplification |
| 2–5% | Moderate noise or minor interference | Secure — monitor; increase sample size |
| 5–11% | Elevated noise; possible partial eavesdrop | Caution — increase error correction overhead |
| ≥11% | Eve likely intercepting > 44% of qubits | ABORT — discard key, restart session |

The 11% abort threshold is derived from information-theoretic security proofs (Mayers 2001; Lo & Chau 1999): below this threshold, privacy amplification can reduce Eve’s knowledge of the final key to an arbitrarily small amount. Above it, the secret-key capacity of the channel drops to zero and no secure key can be extracted.

## 2.4  Real-World QBER Sources (Beyond Eavesdropping)

In practice, QBER has several non-malicious sources that must be characterized and subtracted:

- Detector dark counts: Spurious clicks in single-photon detectors, especially at low photon rates.

- Optical alignment errors: Slight misalignment in polarization optics introduces basis-dependent errors.

- Fiber birefringence: Optical fibers rotate the polarization of photons over distance, requiring active compensation.

- Multi-photon pulses: Practical lasers occasionally emit two photons instead of one, giving Eve a ‘free’ copy via the photon-number splitting (PNS) attack.

In well-calibrated fiber-optic QKD systems (e.g., commercial systems from Toshiba, ID Quantique, QuantumCTek), baseline QBER is typically 1–3%. Any measurement significantly above this baseline warrants eavesdropping investigation.

# Part 3: Quantum Temporal Authentication (QTA)

## 3.1  Concept & Motivation

While QBER detects eavesdropping statistically (requiring enough samples to reach significance), Quantum Temporal Authentication (QTA) provides a complementary physical detection mechanism. QTA exploits the fact that Eve’s intercept-resend attack is not instantaneous — it takes non-zero physical time to measure a quantum state and prepare a new one.

By verifying that each photon arrives within a precise timing window consistent with the channel’s propagation delay, Alice and Bob can flag photons that experienced extra latency — the signature of an interception event. This operates at nanosecond (10⁻⁹ s) resolution.

## 3.2  The Physics Behind the Timing Delay

### Propagation Time Without Eve

For a fiber-optic channel of length L, the expected photon arrival time is fixed by the speed of light in fiber (approximately 2 × 10⁸ m/s, or two-thirds of c in vacuum):

t_expected  =  L / v_fiber  =  L / (2 × 10⁸ m/s)

For a 10 km channel: t_expected = 10,000 m / (2×10⁸ m/s) = 50,000 ns (50 μs). Timing jitter from high-quality single-photon avalanche diodes (SPADs) is typically 0.5–1.0 ns.

### Propagation Time With Eve (Intercept-Resend)

Eve’s intercept-resend attack requires three sequential operations, each taking non-zero time:

- Photon detection: Eve’s detector measures the incoming photon — this collapses the quantum state and takes ~0.5–1 ns.

- Classical processing: Eve records the measurement outcome and decides on a re-preparation basis — ~0.1–0.5 ns with modern electronics.

- Qubit re-preparation: Eve’s source generates a new photon encoding the measured bit in a chosen basis — ~0.5–2 ns.

- Re-transmission: The new photon is injected into the second channel segment toward Bob.

Total extra delay: typically 1–3 ns per intercepted qubit, above the expected propagation time.

For a 10 km channel where the expected transit time is 50,000 ns, an extra 1–3 ns represents a 0.002–0.006% deviation. While tiny, this is well within the detection capability of synchronized atomic clocks or GPS-disciplined oscillators used in modern QKD hardware.

## 3.3  QTA Verification Protocol

QTA is implemented as follows:

- Step 1 — Synchronization: Alice and Bob synchronize their clocks to sub-nanosecond precision using GPS-disciplined oscillators or White Rabbit Ethernet timing protocol.

- Step 2 — Timestamping: Alice records the emission time of each photon with a time-to-digital converter (TDC).

- Step 3 — Arrival logging: Bob’s detector timestamps each photon arrival using a matched TDC.

- Step 4 — Classical comparison: After a batch of qubits, Alice and Bob compare timestamps over the authenticated classical channel.

- Step 5 — Anomaly detection: Any photon with a transit time deviating by more than the QTA window (e.g., ±2 ns) is flagged as a potential interception.

- Step 6 — Threshold decision: If the fraction of timing anomalies exceeds a pre-agreed threshold (e.g., >2% of photons), the session is aborted and the alert is logged.

## 3.4  QTA vs. Classical Timing Verification: A Comparison

| Detection Method | QBER (Statistical) | QTA (Temporal) |
| --- | --- | --- |
| Underlying Principle | No-cloning theorem; basis mismatch errors | Physical propagation delay; intercept latency |
| Detection Speed | After sifting + sampling (~ms) | Per-photon, real-time (~ns) |
| Sensitivity | Eve must intercept >44% to be flagged | Any intercept adds measurable delay |
| False-Positive Rate | Low with large sample sizes | Very low with tight timing windows |
| Channel Required | Public classical channel for comparison | Authenticated classical channel for sync |
| Complementary Role | Primary security guarantee | Secondary / independent alarm layer |

## 3.5  Limitations & Attack Considerations

QTA is a strong secondary defence but has limitations that must be understood:

- Relay attacks: A sufficiently advanced attacker using quantum memory could store a photon, wait, and re-emit it later. QTA windows would need to be tightened, and additional channel authentication steps employed.

- Fiber length variation: Temperature-induced fiber expansion can shift transit times by small amounts. Active temperature compensation or wider QTA windows may be needed for long links.

- Detector timing jitter: Lower-quality detectors with >2 ns jitter may produce false positives. Premium SPADs with <100 ps jitter are standard in modern QKD systems.

- Coherent attacks: A quantum computer-equipped adversary could theoretically perform attacks that partially evade QTA; however, such attacks are prevented by QBER and privacy amplification simultaneously.

# Part 4: Combined Security Framework

## 4.1  Why Both Mechanisms Are Needed

QBER and QTA are orthogonal detection mechanisms that address different aspects of the eavesdropping problem. Relying on only one creates exploitable gaps:

- QBER alone: Eve intercepting only 10–20% of qubits may stay below the 11% abort threshold, remaining statistically invisible while gaining partial key information.

- QTA alone: An attacker with quantum memory could forward photons without introducing timing delays, evading QTA entirely but still disturbing quantum states (caught by QBER).

Together, they create a defence-in-depth architecture:

Combined Security Posture: Eve must simultaneously avoid introducing statistical errors (evade QBER) AND avoid introducing timing delays (evade QTA). These requirements pull in opposite directions: faster interception means more errors; lower error rates require more processing time. Any realistic attack will be caught by at least one mechanism.

## 4.2  Information-Theoretic Security

BB84 with QBER checking achieves information-theoretic (unconditional) security, meaning security holds against adversaries with unlimited computational power. This is fundamentally superior to classical public-key cryptography (RSA, ECC), which could be broken by a sufficiently powerful quantum computer running Shor’s algorithm.

The security proof works as follows: if QBER < 11%, privacy amplification (universal hashing) can compress the sifted key such that Eve’s mutual information with the final key is exponentially small — approaching zero as key length increases.

## 4.3  Simulation Results Summary

The accompanying simulation (bb84_qkd_simulation.py) demonstrated the following key results across 2,000-qubit sessions on a 10 km fiber link:

- No Eve: QBER = 0.00%, 0 QTA violations → session SECURE.

- Eve at 100% interception: QBER = ~24.8% (near theoretical 25%), ~960 QTA violations → session ABORTED on both grounds.

- Critical threshold sweep: QBER crossed 11% at Eve’s interception rate of ~44%, exactly matching the theoretical prediction (0.11 / 0.25 = 44%).

- QTA sensitivity: At 100% interception, ~48% of qubits produced timing anomalies (extra 1–3 ns delay), well above the 2% abort threshold.

# Conclusion

The BB84 protocol, augmented with rigorous QBER monitoring and Quantum Temporal Authentication, provides a multi-layered, physics-guaranteed security framework for quantum key distribution. Its security does not depend on any computational assumption and is provably secure against all known attacks, including those from quantum computers.

QBER leverages the no-cloning theorem and the measurement disturbance principle to make eavesdropping statistically visible. QTA exploits the physical latency of quantum state manipulation to make eavesdropping temporally visible. Together, they ensure that any adversary attempting to intercept Alice and Bob’s quantum channel will inevitably leave a detectable signature, allowing the communicating parties to abort before any key material is compromised.

Fundamental Principle: In quantum cryptography, security is not a promise — it is a law of physics. No adversary, regardless of computational power or technological sophistication, can eavesdrop on a BB84 channel without being detected.

## Key References

- Bennett & Brassard (1984): "Quantum cryptography: Public key distribution and coin tossing" — Original BB84 paper.

- Mayers (2001): "Unconditional security in quantum cryptography" — Formal security proof.

- Wootters & Zurek (1982): "A single quantum cannot be cloned" — No-cloning theorem.

- Gisin et al. (2002): "Quantum cryptography" — Comprehensive review of QKD protocols and implementations.

- Lo & Chau (1999): "Unconditional security of quantum key distribution over arbitrarily long distances".